SCHEDULE I

[See rules 3(e),4(b),7(1),7(2),7(5),7(6) and 8(2)]

Part-1

Biomedical wastes categories and their segregation, collection, treatment, processing and

disposal options

| Category | Type of Waste | Type of Bag or Container to be used | Treatment and Disposal options |
| --- | --- | --- | --- |
| (1) | (2) | (3) | (4) |
| Yellow | (a)Human Anatomical Waste: Human tissues, organs, body parts and fetus below the viability period (as per the Medical Termination of Pregnancy Act 1971,amended from time to time). | Yellow coloured non-chlorinated plastic bags. | Incineration or Plasma Pyrolysis or deep burial* |
|  | (b)Animal Anatomical Waste : Experimental animal carcasses, body parts,organs, tissues, including the waste generated from animals used in experiments or testing in veterinary hospitals or colleges or animal houses. |  |  |
|  | (c) Soiled Waste: Items contaminated with blood, body fluids like dressings, plaster casts, cotton swabs and bags containing residual or discarded blood and blood components. |  | Incineration or Plasma Pyrolysis or deep burial* In absence of above facilities, autoclaving or micro-waving/ hydroclaving followed by shredding or mutilation or combination of sterilization and shredding. Treated waste to be sent for energy recovery. |
|  | (d) Expired or Discarded Medicines: Pharmaceutical waste like antibiotics, cytotoxic drugs including all items contaminated with cytotoxic drugs along with glass or plastic ampoules, vials etc. | Yellow coloured non-chlorinated plastic bags or containers | Expired `cytotoxic drugs and items contaminated with cytotoxic drugs to be returned back to the manufacturer or supplier for incineration at temperature>1200 0C or to common bio-medical waste treatment facility or hazardous waste treatment, storage and disposal facility for incineration at >12000C Or Encapsulation or Plasma Pyrolysis at >12000C.All other discarded medicines shall be either sent back to manufacturer or disposed by incineration. |
|  | (e) Chemical Waste: Chemicals used in production of biological and used or discarded disinfectants. | Yellow coloured containers or non-chlorinated plastic bags | Disposed of by incineration or Plasma Pyrolysis or Encapsulation in hazardous waste treatment, storage and disposal facility. |
|  | (f) Chemical Liquid Waste : Liquid waste generated due to use of chemicals in production of biological and used or discarded disinfectants, Silver X-ray film developing liquid, discarded Formalin, infected secretions, aspirated body fluids, liquid from laboratories and floor washings, cleaning, house-keeping and disinfecting activitiesetc. | Separate collection system leading to effluent treatment system | After resource recovery, the chemical liquid waste shall be pre-treated before mixing with other wastewater. The combined discharge shall conform to the discharge norms given in Schedule-III. |
|  | (g) Discarded linen, mattresses, beddings contaminated with blood or body fluid. | Non-chlorinated yellow plastic bags or suitable packing material | Non- chlorinated chemical disinfectionfollowed by incineration or Plazma Pyrolysis or for energy recovery. In absence of above facilities, shredding or mutilation or combination of sterilization and shredding. Treated waste to be sent for energy recovery or incineration or Plazma Pyrolysis. |
|  | (h) Microbiology, Biotechnology and other clinical laboratory waste: Blood bags, Laboratory cultures, stocks or specimens of microorganisms, live or attenuated vaccines, human and animal cell cultures used in research, industrial laboratories, production of biological, residual toxins, dishes and devices used for cultures. | Autoclave safe plastic bags or containers | Pre-treat to sterilize with nonchlorinated chemicals on-site as per National AIDS Control Organisation or World Health Organisation guidelines thereafter for Incineration. |
| Red | Contaminated Waste (Recyclable) (a) Wastes generated from disposable items such as tubing, bottles, intravenous tubes and sets, catheters, urine bags, syringes (without needles and fixed needle syringes) and vaccutainers with their needles cut) and gloves. | Red coloured non-chlorinated plastic bags or containers | Autoclaving or micro-waving/ hydroclaving followed by shredding or mutilation or combination of sterilization and shredding. Treated waste to be sent to registered or authorized recyclers or for energy recovery or plastics to diesel or fuel oil or for road making, whichever is possible. Plastic waste should not be sent to landfill sites. |
| White (Translucent) | Waste sharps including Metals: Needles, syringes with fixed needles, needles from needle tip cutter or burner, scalpels, blades, or any other contaminated sharp object that may cause puncture and cuts. This includes both used, discarded and contaminated metal sharps | Puncture proof, Leak proof, tamper proof containers | Autoclaving or Dry Heat Sterilization followed by shredding or mutilation or encapsulation in metal container or cement concrete; combination of shredding cum autoclaving; and sent for final disposal to iron foundries (having consent to operate from the State Pollution Control Boards or Pollution Control Committees) or sanitary landfill or designated concrete waste sharp pit. |
| Blue | (a) Glassware: Broken or discarded and contaminated glass including medicine vials and ampoules except those contaminated with cytotoxic wastes. | Cardboard boxes with blue colored marking | Disinfection (by soaking the washed glass waste after cleaning with detergent and Sodium Hypochlorite treatment) or through autoclaving or microwaving or hydroclaving and then sent for recycling. |
|  | (b) Metallic Body Implants | Cardboard boxes with blue colored marking |  |

*Disposal by deep burial is permitted only in rural or remote areas where there is no access to

common bio-medical waste treatment facility. This will be carried out with prior approval from the

prescribed authority and as per the Standards specified in Schedule-III. The deep burial facility

shall be located as per the provisions and guidelines issued by Central Pollution Control Board

from time to time.